MQTT and CoAP exercises (Part 1)

March 13, 2025

Task 0 (Verify connectivity – Does not have to be documented in the exercise report):

Open a terminal window and execute the following commands:

- cd ~/mqtt

- sudo ./netgenerate.sh

The second of these commands creates a small virtual network using Linux network namespaces with three hosts (H1, H2, and H3) connected to a switch, as illustrated in Figure 1. The IPv4 addresses of the hosts are 10.0.0.1 for H1, 10.0.0.2 for H2, and 10.0.0.3 for H3. The command also opens an Xterm window on each host so you can execute commands on a specific host.

Figure 1 – Switched network with three hosts.

Select an arbitrary host and (in that host’s Xterm window) check if you can ping the other two hosts with the command ping -c1 <ip-address>

Notes:

- The virtual network created with the sudo ./netgenerate.sh command disappears if the virtual machine is restarted, so you will need to execute the commands at the start of Task 0 to recreate it after a restart of the VM.

- If the virtual network is not needed anymore, it can be deleted with the script clearnet.sh in the mqtt directory by executing: sudo ~/mqtt/clearnet.sh

Task 1 (Basic MQTT connections):

In H1’s Xterm window, type ip addr and verify that one of its interfaces has an IP address in the 10.0.0.0/24 range. Type the following command in H1’s Xterm window to start the MQTT-broker on this host: sudo ./broker.sh start

Start Wireshark in the VM’s normal terminal window ( sudo wireshark ) and start a capture on the veth11 interface. It may be a good idea to set the display filter (just below the toolbar at the top of the window) to mqtt as shown in Figure 2:

Figure 2 – Wireshark with mqtt as display filter.

In H2’s Xterm window, start an MQTT-client as subscriber:

- mosquitto_sub -v -h <H1’s IP-address> -t '#'

Stop the client again after a few seconds with Ctrl-C in the Xterm window, switch to the Wireshark window and stop the capture. You should have captured five MQTT packets; list these packets and explain their purpose.

Start a new capture with Wireshark and restart the subscriber client again (as above), but keep it running for at least 2 minutes – what do you observe in Wireshark? Can you relate the additional messages and their intervals to the contents of one of the earlier packets?

Start a new Wireshark capture while the subscriber client is running. In H3’s Xterm window, publish a message with: mosquitto_pub -h <H1’s IP-address> -t Topic1 -m "Hello World!"

Verify that your message appears in H2’s Xterm window. Switch to Wireshark and stop the capture. List and explain the MQTT packets related to the publishing of the message. Do the MQTT “Publish Message” contents correspond to the message string of the mosquitto_pub command? (Hint: Check the “raw” bytes of the message in Wireshark – if the ‘Packet Bytes’ section of Wireshark’s window is not shown, make sure that ‘Packet Bytes’ is selected in the View menu.)

Task 2 (Message QoS):

MQTT supports three different “QoS”-levels related to message delivery, but only the default QoS level (level 0) was used in Task 1. A non-default QoS-level can be specified for both the MQTT-subscriber and the MQTT-publisher by appending the option -q <qos-level> (where <qos-level> is 1 or 2 ) to the commands. For each non-default QoS level, capture MQTT traffic with Wireshark as in Task 1 and explain the difference between the different QoS levels 0, 1, and 2 (It may be helpful to consult section 4.3 in the official MQTT-specification – see https://mqtt.org).

Also, capture and explain the MQTT traffic if the publisher uses QoS-level 0 but the subscriber has requested QoS-level 2.

Task 3 (Wills):

MQTT permits a publisher to set up a “will”, i.e., a message to be sent in case the publisher unexpectedly disconnects from the broker. In Tasks 1 and 2, the publisher disconnected immediately after a single message, but the publisher can also be instructed to repeat a message a specified number of times with a specific interval between the messages. Execute the following command in H3’s Xterm window and verify/document with Wireshark that: A) the message is sent 5 times with 2 seconds intervals between each message, and B) the publisher disconnects after the 5th message has been sent:

mosquitto_pub -h 10.0.0.1 -t Topic1 -m "Hello World!" --repeat 5 --repeat-delay 2

Consult the documentation for the mosquitto_pub command (found on https://mosquitto.org) and determine the extra arguments that must be added to the command so that the publisher creates a “will” with the text “Help me!” to be published to the topic “Wills” if it disconnects unexpectedly.

Start a Wireshark capture. Rerun the publisher command with these extra arguments and the value 60 for the --repeat-delay option, so the command runs for at least 4 minutes.

Execute (in a new terminal window of the VM) the command (this will forcefully kill the publisher):
sudo pkill mosquitto_pub

Document in your report with the Wireshark capture and a screenshot of the subscriber’s Xterm window that the “will” is sent to the subscriber.

Task 4 (Retained messages):

Normally, an MQTT client that starts subscribing to a topic will not receive anything before a message is published to that topic. However, MQTT also supports “retained” messages. If a publisher indicates that messages published on a topic should be retained, the broker keeps a copy of the last published message on that topic, so if a client starts subscribing to this topic, it will receive the retained message immediately.

Setup and investigate two scenarios, one with and one without retained messages, illustrating the difference between the scenarios, and document these with Wireshark captures. You may also need to consult the documentation for the mosquitto_pub command for this task.

Note that if you want to “unretain” a message on the broker, you must publish an empty message (also called a null message) to be retained, i.e., use -m '' instead of -m 'Hello world!' as an argument for mosquitto_pub. You may also use -n instead of -m ''

Task 5 (Persistent client mode):

Normally, when the subscriber connects it starts a new session with new subscriptions. However, the subscriber may instruct the broker to enable a persistent client mode where the broker remembers the subscriber’s session after the subscriber disconnects, i.e., its subscriptions and any QoS 1/2 messages that arrive after the subscriber disconnects. When the subscriber later reconnects, it should receive any stored QoS 1/2 messages.

Setup a scenario that demonstrate the persistent client mode where several QoS 1 messages are sent to the subscriber when it reconnects and resumes a previous session. The documentation should include a Wireshark capture that illustrates how several (at least 10) stored QoS 1 messages are sent to the subscriber when it reconnects and how the subscriber acknowledges these.

Hints:

- When you start / restart the subscriber, you may need the -i sessionid option (where sessionid is just some arbitrary string, e.g., 0123456789, so that the broker knows that the two invokations of the subscriber client are part of the same session.

- If you capture on the veth13 interface in Wireshark, you will only capture the packets between the switch and H3, i.e., you will only see the packets between the broker and the subscriber.

The broker can similarly be stopped with sudo ./broker.sh stop and you can check the status with sudo ./broker.sh status

I.e., not in any of the Xterm windows of the hosts.

The veth11 interface is the switch’s interface towards host H1

Neither scenario should use wills, i.e., the options for wills identified in task 3 should not be used in task 4.