MQTT and CoAP exercises (Part 2)

March 20, 2025

Task 0 (Verify connectivity – Does not have to be documented in the exercise report):

Open a terminal window and execute the following commands (note the extra option to the netgenerate.sh script, which specifies that the simulated nodes use IPv6 addresses instead of the default IPv4 addresses):

- cd ~/coap

- sudo ./netgenerate.sh --ipv6

The second of these commands creates a small virtual network using Linux network namespaces with three hosts (H1, H2, and H3) connected to a switch, as illustrated in Figure 1. The IPv6 addresses of the hosts are fd00::1 for H1, fd00::2 for H2, and fd00::3 for H3. The command also opens an Xterm window on each host so you can execute commands on a specific host.

Figure 1 – Switched network with three hosts.

Select an arbitrary host and (in that host’s Xterm window) check if you can ping the other two hosts with the command ping -c1 fd00::x (where x is 1, 2, or 3, depending on the host).

If the virtual network is not needed anymore, it can be deleted with the script clearnet.sh in the coap directory by running the following command (in a terminal):  sudo ~/coap/clearnet.sh

Note that the virtual network created with the sudo ./netgenerate.sh --ipv6 command disappears if the virtual machine is restarted, so you must execute the commands at the start of Task 0 to recreate it after a VM restart.

Task 1 (Basic CoAP):

Start a CoAP server on host H1 with the command (in H1’s Xterm window):

coap-server -A fd00::1

Start Wireshark in the normal terminal window with sudo wireshark and start a capture on the veth11 interface with a display filter set to coap.

Task 1a: Use the CoAP client program in H2’s Xterm window to retrieve the default resource (‘/’) from the server with the command coap-client coap://[fd00::1]/

Stop the capture in Wireshark and examine the contents of the packets captured:

- Which UDP-port is the coap-server listening to?

- What version of CoAP is used?

- Does the message exchange use confirmable or non-confirmable messages?

- How many options are included in the CoAP message sent from fd00::1 to fd00::02? What are the purposes of these options?

Task 1b: Start a new Wireshark capture and use the coap-client to retrieve the ‘/async’ resource with the command coap-client coap://[fd00::1]/async 
Stop the Wireshark capture. Explain why the retrieval of the ‘/async’ resource in this task resulted in a different sequence of CoAP messages compared to task 1a.

Task 1c: Start a new capture in Wireshark and use the CoAP client program again to retrieve both the default resource (as in task 1a) and the ‘/async’ resource (as in task 1b), but now with the option ‘-N’ between the command and the URI.

Explain the difference in terms of the CoAP messages in tasks 1a, 1b, and this task.

Task 2 (Dynamic resources):

Stop the server (press Ctrl-C when H1’s Xterm window has focus). To enable that new resources can be added to the server, the server must be restarted with an extra option: -d 9

The extra option permits the clients to define and set the values of up to 9 dynamic resources.

Task 2a: Start a new capture in Wireshark. Use the coap-client in H2’s Xterm window to define a resource with the name C34351 (i.e., the URI will be coap://[fd00::1]/C34351 ) and the value “DTU’s best course”. You will need to use additional options for the CoAP client compared to task 1 to define:

- The action/method that creates/instantiates a dynamic resource on the server.

- The value to be stored in the resource.

- The type of information for the value.

Please consult the documentation for the coap-client to determine which options are required and how they are used. Document that the dynamic resource is created with the capture from Wireshark.

Task 2b: Use the client to retrieve the resource with URI coap://[fd00::1]/C34351 again. Does it contain the expected value?

Task 2c: Now use the client to delete the resource and verify afterwards that it has been deleted.

Task 3 (Observing a resource):

Start a new wireshark capture. Use the client to observe / subscribe to (Hint: consult the client documentation again) the URI coap://[fd00::1]/time for 15 seconds. Stop the Wireshark capture and explain the CoAP packets captured. In particular, explain how the server knows when to stop sending messages to the client after this has stopped observing the resource.

Task 4: (Publish/Subscribe with CoAP):

Design a setup with one server and two clients that resembles an MQTT network, i.e., with one client acting as a publisher that (in this exercise manually) “publishes” new text content to a dynamic resource of your choosing on the server and the other client acting as a subscriber that “subscribes” to this dynamic resource for a suitable time.

List the commands you use to make this setup and document with a Wireshark capture and screenshots that the setup works as expected when you manually publish several (i.e., more than one) text messages. Note: You should probably run the server command on H1 (and capture the coap traffic on interface veth11) and run the clients on H2 and H3.

Task 5:

Stop the CoAP server in H1. Start new CoAP-servers on both H1 and H2 with the command: coap-server -g ff05::fd

Start a Wireshark capture on interface veth31. Execute the following command in H3’s Xterm window: coap-client -N coap://[ff05::fd]/ -s 15

Stop the Wireshark capture and explain the CoAP packets captured, including:

- What is the type of MAC address used as the destination MAC address in the CoAP packet from the client?

- Why is the ‘-N’ option needed in this scenario?

Restart a Wireshark capture on the same interface as before, but this time without a display filter. Repeat the command for the coap-client but this time without the ‘ -s 15 ’ option. Stop the Wireshark capture and explain the different ICMPv6 messages.

Task 6:

Stop the CoAP servers on H1 and H1. Start a CoAP server on H1 again as in Task 1. Use a CoAP-client on H2 to retrieve (from the CoAP-server on H1) the resource /.well-known/core

Explain the output from the client.

Note that when IPv6 addresses are used directly, the address is enclosed in square brackets, ‘[‘ and ‘]’.

You can ignore that Wireshark describes an option as “Unknown Option” – this is only because the installed version of Wireshark has not been updated to “recognize” all possible CoAP message options.

You may need to consult RFC-7252 (https://www.rfc-editor.org/rfc/rfc7252) for a description of CoAP options.

See https://libcoap.net/doc/reference/4.3.4/man_coap-client.html

RFC-7641 (https://www.rfc-editor.org/rfc/rfc7641) might be relevant to consult.