Mininet-WiFi exercises (Part 1)

February 20, 2025

General tips:

- Resetting Mininet-WiFi (good practice if emulator aborts):

  - Linux CLI: sudo mn –c

- Disable Linux’s built-in network-manager (should be done every time the VM is started for the Mininet-WiFi exercise):

  - Linux CLI: sudo systemctl stop NetworkManager.service

- Saving Mininet-WiFi output (but unfortunately not your input) in a file (here outputfile.txt):

  - Linux CLI: sudo python myscript.py 2>&1 | tee outputfile.txt

- You can use the up and down arrow keys on your keyboard to quickly scroll up and down in your command history. This works for both the Linux CLI and the Mininet-WiFi CLI.

Step 1:

Open a terminal and change directory to the mininet-wifi directory with: cd mininet-wifi

Verify that the directory contains a file with the name skeleton.py using the ls command. Make a copy of the skeleton.py file with: cp skeleton.py network1.py

Modify network1.py so that you have a small WiFi network with an access point (called ap1 using 802.11g) and two wireless stations (called sta1 and sta2) – use the information at the end of this document for information about the relevant methods and arguments. Ensure that the stations have unique IP addresses in the same IP subnet. Position the two stations so they are no more than 5 meters from the access point.

Figure 1 – Network for step 1.

Run the emulator with: sudo ./network1.py

In the graph that opens, what do you think the large elipses represent? Document that both stations are associated with the access point. To verify that, e.g., sta1 is associated with the access point, you can use the command sta1 iwconfig at the Mininet-WiFi CLI (and similarly for sta2) – how can you see from the output of the iwconfig command that a station is associated with the access point. Also document that the two station can communicate by ping’ing sta2 from sta1 (as last week: sta1 ping -c1 sta2)

Step 2:

Test the throughput between the two stations using the iperf program like last week. Reposition the two stations so that they are close to the edge of the access point’s coverage area (hint: Maybe 40 m is a suitable distance?). Repeat the throughput measurements. Compare and discuss your results (remember that the maximum physical bit rate for an 802.11g network is 54 Mbit/s).

Step 3:

Create a copy of network1.py named network2.py. Modify network2.py to add a wired host (called h1) to the network and connect it to the access point with a (wired) link. Document that the wireless stations and the wired host can communicate using the Mininet-WiFi command: pingall

Figure 2 – Network for step 3.

Measure the throughput between one of the WiFi stations, e.g., sta1, and h1 – are the values the same as between the two WiFi stations? (Why / why not)? You must test both “directions”, i.e., both when the iperf server (iperf –s) is running on h1 and the iperf client (iperf –c server) is running on sta1 and the opposite configuration. Create a table like the one shown below and comment on the results.

|  | Throughput reported by server |
| --- | --- |
| Iperf server on h1 |  |
| Iperf server on sta1 |  |

Step 4:

Create a copy of network2.py named network3.py. Modify network3.py to add an additional wired host. You will need to add a switch to the network that connects to the access point and to the two wired hosts.

Figure 3 – Network for step 4.

Verify that the network is fully connected with the pingall command. Also verify that the throughput between a wireless station and a wired station is as expected.

Measure the throughput between the two wired hosts and comment on the result, especially if it seems unreasonably large (hint: Remember you’re emulating a local area network, not measuring on a real physical network with physical devices, cables, etc.)

Step 5:

Start network1.py again. Enable the hwsim0 interface with ap1 ifconfig hwsim0 up at the Mininet-WiFi CLI. Start Wireshark from a new terminal window (sudo wireshark) and start a capture on the hwsim0 interface and stop it again when you have captured 5-10 beacon frames.

How often (approximately) are beacon frames broadcasted? What bit rates are supported by the access point? What bit rate is the beacon frame transmitted with and why?

Start a new capture with Wireshark on the hwsim0 interface. Execute the following two commands in the Mininet-WiFi CLI:

- link ap1 sta1 down

- link ap1 sta1 up

Stop wireshark and examine the packets captured and look for packets related to the two “link ap1 sta1 ...” commands (you should probably use a display filter in Wireshark to ‘suppress’ the beacon frames – see below). Explain what happens when the commands are executed – it may help to look at the contents (MAC addresses) of the Source and Destination columns in Wireshark to see the ‘direction’ of a frame, e.g., from the access point to one/all stations or the other direction.

Notes:

- You can use a display filter in Wireshark to hide packets you’re not interested in. E.g., the display filter wlan.fc.type == 0 makes sure that you only see management packets. If you want to see management packets except for beacons, you can use:
(wlan.fc.type == 0) and (wlan.fc.subtype != 8)

- To get the WiFi MAC address of a station, use the command sta1 ifconfig at the Mininet-WiFi CLI and look for the ‘ether xx:xx:xx:xx:xx:xx’ part.

- To get the MAC address of the access point that a station is connected with, use the command sta1 iwconfig at the Mininet-WiFi CLI and look for the
‘Access point: xx:xx:xx:xx:xx:xx’ part.

Some important methods of the Mininet_wifi class:

The following notation is used:

- methodName(arg1, arg2, ..., argn, ...)

  - arg1, arg2, ..., argn are normal (positional) arguments, i.e., values for all has to be provided

  - The remaining arguments (indicated by the second '...') are named arguments so they have to be provided in the form of 'name1 = <value_of_name1>' (i.e. 'argumentname = argumentvalue') and they must come after the normal arguments.

- Example: if a method is given as methodName(name, address, ...) it can be called as

  - methodName('John Doe', 'Main Street 1', zipcode = '1234', country = 'US')
where two named arguments (zipcode and country) are provided in addition to the two normal arguments.

Methods in the Mininet_WiFi class for WiFi nodes

- addAccessPoint(name, ...)

  - name: (String) internal name of the access point, used to distinguish between different APs

  - <ssid> : (Mandatory) SSID of the WiFi network, e.g., ssid = 'my-ssid'

  - <mode> : (Mandatory) WiFi mode, single character such as 'a', 'b', 'g', 'n', ...

  - <channel> : (Mandatory) WiFi channel as a string, e.g., channel = '1'

  - <mac> : (Optional) MAC address of the AP in the form 'xx:xx:xx:xx:xx:xx' where the x’es should be replaced by valid hexadecimal digits. A random (unique) MAC address is assigned if this argument is not provided.

  - <position> : (Mandatory) Position in 3D-space as a string 'xpos,ypos,zpos', e.g., '70,30,0' means an x-position of 70 m, y-position of 30 m, and z-position (height) of 0 m.

  - <freq> : Which frequency band to use, either '2' (for 2.4 GHz) or '5' (for 5 GHz) – only needed if the <mode> argument specifies a WiFi technology that can use both bands.

- addStation(name, ...)

  - name

  - <mac>: (Optional), as for addAccessPoint()

  - <ip>: (Mandatory). IP address of the station in CIDR-notation

  - <position>: (Mandatory), as for addAccessPoint()

Methods in the Mininet_WiFi class for Wired node:

- addHost(name, ...)

  - name

  - <ip>: (Mandatory)

  - <mac>: (Optional)

  - <position>: (Optional and also not shown in the graph)

- addSwitch(name, ...)

  - name

  - <position>: (Optional and also not shown in the graph)

Methods in the Mininet_WiFi class for wired links:

- addLink(device1, device2)

You can use a GUI editor with: mousepad network1.py

Make sure the host h1 has a unique IP addresses in the same IP-subnet as the wireless stations.

Note that wired hosts are not shown in the graph that opens when you start the network.

Remember to start the switch just like you would start an access point.

Note: You’ll have to combine information from two parts of “Wireless Management” part of the beacon frame to answer this.

Hint: Look in the “802.11 radio information” section of the wireshark capture.

Remember that CIDR-notation includes the size of the network-part of the address, e.g., “192.168.0.24/16”