Mininet-WiFi exercises (Part 2)

General tips:

- Disable Linux's built-in network-manager (should be done every time the VM starts):

  - Linux CLI: sudo systemctl stop NetworkManager.service

- Saving Mininet-WiFi output (but unfortunately not your input) in a file (here outputfile.txt):

  - Linux CLI: sudo python myscript.py 2>&1 | tee outputfile.txt

- Resetting Mininet-WiFi (good practice if the emulator terminated incorrectly):

  - Linux CLI: sudo mn –c

- You can use your keyboard's up and down arrow keys to quickly scroll up and down in your command history in the Linux CLI and the Mininet-WiFi CLI.

Step 1:

Create a copy of the skeleton.py file in the mininet-wifi directory and edit the copy to create the network in Figure 1, where the station is placed approx. 20 meters from the access point. The access point should be an 802.11a access point (i.e., in the 5 GHz band) configured to use channel 36. When you create the station, you should include the argument mode = 'a' to indicate that it should also use 802.11a mode.

Figure 1

Note that in the first line of the topology() function, the argument wmediumd_mode=interference must be changed to wmediumd_mode=error_prob

Furthermore, after the line net.configureWifiNodes() you must add the line
net.addLink(sta1, ap1, error_prob = 0.0)

Start Mininet-WiFi with the new file and verify that sta1 is associated with the access point using the iwconfig command on sta1 – note that it may take some seconds for sta1 to become associated with the access point, so you may need to repeat the iwconfig command a couple of times. Then use the pingall command to verify that sta1 and h1 can communicate.

Use iperf to measure the throughput from sta1 (as iperf client) to h1 (as iperf server). Repeat the throughput measurements when the error_prob argument in the net.addLink() method above is subsequently set to 0.001, 0.003, 0.01, 0.03, and 0.1, respectively. Unfortunately, you must exit Mininet-WiFi, edit the *.py file with a new value, and restart Mininet-WiFI between every value of the error_prob argument.

Make five measurements (i.e., execute the iperf command five times on the client) for every value of error_prob so that you can calculate an average value.

Make a plot of the average throughput as a function of the error probability and comment on the results.

Note that the iperf command may fail and cause the emulation to abort if there are too many errors when you execute the iperf command. In this case, you should exit the emulator, reset it with the sudo mn -c command, restart it, and attempt again.

Step 2:

Create the network in Figure 2 in Mininet-WiFi. The access points should use the 802.11a variant on channel 36, as in the previous step. If you use a copy of your topology from the previous step as basis, change the wmediumd_mode argument back to wmediumd_mode=interference (as in step 1) and remove the net.addLink() commands for the wireless links.

Figure 2

The two access points' coverage areas are disjoint so that when Sta1 moves (as indicated by the dotted arrow) from the left BSS to the right BSS, it will be out of coverage for some time. You may need to experiment with the positions of the two access points so their coverage areas do not overlap and you have time to execute commands on Sta1 when it’s between the coverage areas.

Execute the network in Mininet-WiFi and document in your journal:

- That Sta1 is initially associated with AP1 and that it can ping h1.

- That Sta1 is not associated with any AP when it is between the BSSs.

- That Sta1 is eventually associated with AP2 and can (again) ping h1. You will sometimes have to wait about 60 seconds before the flow through AP1 between Sta1 and h1 times out in the switch, before you can ping via AP2. If, after more than 60 seconds, you are still unable to ping between Sta1 and h1, use the dpctl del-flows command in the Mininet-WiFi CLI.

Tip: First create the network without mobility and verify that Sta1 can ping h1. Then add Sta1's mobility for the experiments.

Notes:

- Mininet-WiFi attempts to animate the movements of Sta1, but it is not always smooth, so you might need to repeat the experiment if you want to execute a command on Sta1 when it is, e.g., between the coverage areas.

- If you experimented with mobility, you may get Python-related error messages (which can be ignored) in the Linux console when you exit Mininet-WiFi. To be on the safe side, if you see these errors, use the sudo mn -c command before you start Mininet-WiFi again.